ICG360 gyro documentation for inclusion with CSM simulator V10.

This is a copy of the instructions provided with the real ICG360 gyro - the tail rotor gyro that is simulated in the CSM simulator V10. Please note that the simulation of servo performance has been simplified and the servo speed parameter is omitted from the gyro simulation. Since servo performance, linkage slop, friction, etc affect the stability of the tail rotor system the maximum gain that may be obtained in the simulator is not a good predictor for the gain that may be run in practice.

Note: This gyro is like no other - please read this manual fully before installing and flying the gyro

To bring you a great gyro CSM simply threw away the rule book and started with a clean sheet. The result, the ICG360, is packed with new concepts in gyro technology:-

·	Yaw Rate Demand

·	Flight Modes

·	Heading Lock

·	Built-in Exponential

·	Anti-drift Temperature Calibration

·	Two stage Power Supply regulation

·	Flight mode tailoring via (optional) PC interface cable/software

Introduction

If you are familiar with conventional tail rotor gyro systems it is important to understand how this gyro is different in order to get the best out of it.

Conventional Gyro systems

In a conventional gyro the pilot applies a rudder command and, as the helicopter responds to the command the gyro opposes the pilot's command. The yaw rate achieved in such a system depends on the 'gain' of the gyro. The more gain the gyro has the lower the yaw rate that can be achieved. It is common with such systems to have to reduce the gyro gain to achieve the required yaw rate for some manoeuvres (this being accomplished with the gyro gain switch).

The ICG360

In designing the ICG360 we have adopted a 'Yaw Rate Demand' philosophy making the system a true Yaw rate gyro. In this system the rudder command from the pilot is interpreted as a request to the gyro to establish the desired yaw rate. The gyro drives the tail rotor servo as needed to obtain this yaw rate. This means that the ICG360 makes full yaw rate available even at high gain settings. With this system you can now use the rudder travel volume (ATV) and rudder rates facilities to set up the desired full-stick yaw rate and the gyro gain switch now becomes a 'Flight Mode Switch' for the gyro. There is no need for so called Pilot Authority Mixing - indeed you should not use pilot authority mixing with this gyro

Flight Modes

The ICG360 has two flight modes selectable by the transmitter gyro gain switch (or Auxiliary channel switch).  Note: A rotary knob or slider is not recommended as it is not possible to accurately and repeatedly set the gain with these. The modes are:

Mode 0 (Standard Mode)

This mode gives flying characteristics that are similar to conventional systems.

Mode 1 (Heading Lock Mode)

This mode provides a much higher resistance to unwanted yawing movements than can be obtained with a conventional gyro system. This mode has great advantages in the following situations:

1) For the beginner, where this mode makes it possible for him/her to almost ignore the tail rotor control in the initial hovering phase, which is made even easier as this mode allows the tail trim to be set before the helicopter leaves the ground. (see 'Trim Adjustment')

2) For cross-wind hovering manoeuvres where the natural tendency of the helicopter to weathercock into wind needs to be resisted.

3) For backwards flight manoeuvres and general "3D" flying.

The Standard mode may be employed for basic forward flight where the natural tendency of the helicopter to weathercock can be helpful, especially to the less experienced pilot.

The graph (Figure 1) shows the way in which the gyro gain channel provides both mode switching and independent gain adjustment of the two modes. If the gyro gain channel pulse is longer than the centre value (1.5ms) the gyro is in Standard mode while with the gain channel pulse shorter than 1.5ms the gyro is in Heading Lock mode. The Travel Volume (ATV) settings for the two switch positions provide a convenient way of adjusting the gyro gain for each mode from the transmitter. Increasing the ATV of the gyro gain channel increases the gain for that mode. If it is being used with a basic radio system without a suitable channel for controlling the gyro gain then the gyro will run as a single rate unit with gain adjustable from the manual gain control on the unit itself. In this case the gyro defaults to the Standard mode.

µ §

Figure 1. The Gyro gain channel acts as both gyro mode switch and gain control

Installation.

Siting of the gyro.

µ §

Figure 2. Mounting the gyro

The ICG 360 should be mounted in the helicopter with its axis of rotation (as marked on the gyro case) accurately parallel to the main shaft of the helicopter. The gyro sensor (together with the electronics of the gyro) is anti-vibration mounted inside the gyro case. However, to provide further vibration and shock resistance it is important that the gyro be mounted to the airframe using two of the double sided adhesive foam strips provided. Do not use any other type of mounting foam as this will reduce the performance of your gyro. Replacement strips are available as a CSM spare. The gyro case may be aligned along or transverse to the length of the helicopter. For good adhesion, ensure that the surface to which the gyro is attached is smooth, hard and clean. As with all high performance solid state gyro systems, the ICG360 performs best if sited at a point of low vibration in the helicopter airframe. Where possible avoid siting the gyro at the extreme front of the radio tray for example as this area is often subject to high levels of main rotor generated vibration. Note: Tests suggest that the X-Cell plastic radio tray may be too flexible and that better performance can be gained by mounting the gyro at the rear of the frames.

Connecting to your radio system.

The ICG360 is designed to work with the following radio control systems:-

·	Futaba PPM

·	Futaba PCM 1024

·	JR PPM

·	JR ZPCM

·	JR SPCM

If you wish to use this gyro with other types of radio system please contact CSM for advice.

Connect the gyro to your receiver as indicated in the following table:-

µ §

Connect the "SERVO" output to the tail rotor servo.

Leave the "COMPUTER" connector unconnected. (This is for use with the optional PC interface cable/software). Do not connect any form of extension to this connector.

If you are using a radio system (e.g. a 4 channel aero set) that does not have a channel that can be allocated to the gyro gain/mode then the "Aux" input to the gyro should be left unconnected. In this case the gyro will default to the Standard mode with the gain adjusted by the rotary gain control (near to the  "gyro set" light on the front panel of the gyro). Initially set the gain control to its mid position. Final adjustment will require flight testing.

Futaba 9ZHP/ZAP

On these transmitters it is important to ensure that in all flight modes the gyro sense mode is set to INHIBIT (see the "GYR" entry of the "helicopter condition" menu). Check under the "PMX" entry that no throttle to rudder or rudder to gyro mixing is active in any of the flight conditions. Check in the "P->R" entry that pitch to rudder mixing is inhibited. Under the "model" menu use the "FNC" entry to allocate the gyro control to a switch of your choice.

JR PCM10S/SX

On these radios it is easier to disable the JR 'code 44' (gyro sense adjust) and control the gyro gain by a switch, but if the ability to automatically switch between both gyro modes by the use of the flight mode switch (idle up) is required, then the JR gyro sensing (code 44) should be used. Because the ICG360 gain control works from the centre of the channel outwards, an LCD value of 50% (the channel centre) is in reality zero gain. Increasing the value will increase the gain in one flight mode, the other mode is adjusted by decreasing the value below 50%: be aware that this value will be reversed so that an LCD value of 25% will actually be a higher gain value than a LCD value of 30%. Note: if the gyro is adjusted through the JR gyro software, the gyro channel ATV's/TA should be set to default values (100%).

JR X388S/X3810/X8103

On these radios it is easier to control the gyro gain through the gear channel as it is not always possible to have control over other channels on a 2 position switch. Note: if so desired it is possible to set the gyro up using the JR gyro software on the 3810/8103, see the PCM10 instructions above.

Reversing link

As with any gyro it is vital to ensure that it operates in the correct sense. Failure to do this will cause an uncontrollable pirouette on take-off and beginners who are in any doubt on this aspect of the set-up should seek advice. First establish that the sense of the transmitter control is correct (i.e. that the application of a right rudder command causes a change in tail pitch that will rotate the helicopter to the right). Once this has been done check that turning the helicopter to the left makes the gyro apply a tail rotor command to the right. Should the gyro operate in the wrong sense use the gyro "REVERSE" link to correct it. The gyro is supplied with the reversing link already fitted and to remove it simply pull on its protruding tab and store it safely. When replacing the reversing link ensure that it is fitted over both pins and pushed firmly into full engagement.

Initial Radio set-up

For initial flights we suggest that you adjust the radio system as follows:

ATS (Automatic Tail Stabilisation):  		OFF

Pilot Authority Mixing:  				OFF (this should never be used with the ICG360)

Rudder channel ATV  (both directions):  		50%

Gyro gain channel ATV (both directions):	50%

Mechanical

The ICG360 will work with any servo in the Futaba or JR ranges but as with any gyro system a fast servo (e.g. Futaba FP-S9203 or JR 2700G) will give the best results.

To prevent damage due to excessive servo travel the servo movement is limited by the gyro electronics. (Under no conditions will the servo travel past the electronic limit set by the gyro, regardless of any increase in the ATV value.) You should use the longest servo arm that gives full movement of the tail rotor pitch linkage without causing the linkage to bind or the servo to become stalled at either extreme of travel.

Attention to the tail control linkage is important to getting the best from this gyro. You should aim for a easy-moving but slop-free linkage between the tail servo and the tail blades. Inspect the bearings/thrust races in the tail hub for smoothness of operation. Check the pitch slider and ball links for slop and replace if needed. Some helicopters that have noticeable 'give' in the tail linkage may benefit from the addition of a rear-mounted tail servo and rigid pushrod.

Remember that, during aerobatic manoeuvres, the combination of a high performance gyro and a fast tail servo can place very high loads on the tail rotor drive train. Regularly Inspect tail rotor gears, etc. to ensure that they are in good condition.

Batteries, power consumption, and wiring

Although the power consumption of the gyro unit itself is very modest, as with all high performance Solid State gyro systems the speed of the gyro response will work the tail rotor servo harder than slower mechanical gyro systems. Especially where a high performance servo is being used the battery drain from the tail rotor servo can be high. We recommend that you use a good quality battery state monitor and check it carefully before each flight.

Your receiver battery is a vital part of your tail rotor system. Remember that a battery in a low state of charge or an old battery which has developed a high internal resistance will adversely affect servo performance and may even cause the tail to wag on an otherwise well set up helicopter. You may wish to consider maintaining the charge in your receiver battery between flights by the use of  a quality Delta Peak type field charger. In installing the gyro also bear in mind that voltage drops down long servo extension leads will also detract from servo performance. Where the installation requires extensions to be used (either between the receiver and gyro or between the gyro and the servo) avoid using ones that are unnecessarily long.

Trim adjustment

Initially zero the rudder trim and any rudder sub-trim that your transmitter may have. Next, identify which gyro gain switch position gives you Standard mode and which gives you Heading Lock mode. This can be seen from the difference in the tail rotor servo behaviour on the ground. In Standard mode the rudder servo will return to the neutral position when any rudder command is released, while in heading lock mode the servo will tend to remain at or near its travel limit when a full stick rudder command is applied and then removed. Now set the rudder trim so that with the gyro in Heading Lock mode the servo, once centred, has no marked tendency to creep in either direction (though some slow residual creep of the servo is quite normal). Once this trim position has been found no further adjustment of the transmitter trim should be needed. However, slight adjustment of the helicopter tail control linkage may be needed in order to remove any offsets in the Standard mode. This can only be done by flight trials.

Behaviour on the ground

For those used to conventional gyro systems the behaviour of the ICG360 on the ground may seem unusual.

Unlike conventional systems, it is quite normal for the ICG360 in Standard mode to provide full tail rotor servo movement for rudder commands that are as little as 30% of the full stick movement. This is a consequence of the Yaw rate demand feature.

With the helicopter on the ground and the ICG 360 set in Heading Lock mode it is quite normal for the tail rotor servo to slowly creep and may over the course of say 15 seconds creep to full travel. Your transmitter rudder trim will affect the direction and speed of this creep but even after the trim has been adjusted to minimise this some slow movement is still to be expected. This apparently strange behaviour is caused by the stationary helicopter not responding to the heading corrections requested by the gyro. In the absence of a response from the helicopter the gyro continues to increase the servo command in an attempt to get the helicopter to obey. In flight the helicopter will, of course, respond to the tail servo movements and the system act normally.

Trouble shooting

If the gyro does not respond as expected to any adjustment of the ATV, it may be helpful to plug a spare servo directly into the appropriate RX channel as a check to make sure that the radio is driving the channel as required, and that no other switches, mixers or other functions are interfering with the operation of the channel.

Flying the gyro

Turn-on sequence

1. Turn on your transmitter

2. With the model stationary on the ground turn on your receiver.

3. Wait for about 7.5 seconds keeping the model still while the gyro goes through its self test.

4. Observe that the gyro set light comes on (or if this is inaccessible in your model move the rudder stick full travel in both directions and ensure that the tail rotor servo responds to the stick movements)

5. Your ICG360 is now ready for flight.

If the set light fails to come on or flashes turn off the receiver for a few seconds and repeat the turn on sequence.

!! Make sure the model is not moved during the gyro self test period. !!

Initial flight trials and setting up.

First select flight mode 1 (Heading hold mode) and hover the helicopter. Use short  small 'stabs' of rudder control to disturb the helicopter in yaw and observe. If some tendency to oscillate is seen, slightly reduce the gyro gain. Conversely if no tendency to oscillate is seen try increasing the gain. You are looking at this stage for the highest gain that gives no sign of oscillation when the tail is disturbed by sudden changes in tail command. Observe any trim offset in the tail and correct this with the transmitter rudder trim.

Now switch to Flight mode 0 (Standard mode) and repeat the exercise but in this case any trim offset should be removed not by use of the transmitter trims but by adjusting the tail rotor linkage.

Once this has been done you may wish to check for tail wagging in fast forward flight. Should this be observed, you may wish to lower the gyro gain slightly. Caution should be exercised over the use of the heading hold mode in flying circuits or other general flying until you are familiar with the very special handling characteristics this mode gives.

Adjusting the stick response

After initial setting use the rudder ATV, Rates, and Exponential facilities of the transmitter to tailor the control response as required. Beware that the maximum yaw rate available (at any gain) is very high. If an increase in available yaw rate is required increase the rudder travel on your transmitter gradually until the desired response is obtained. Remember that increasing the ATV/TA will not increase the overall tail throw.

The ICG360 has built-in exponential. You may find the following graph of yaw rate against rudder command useful in deciding on the setting of rudder rates and rudder ATV. Remember that the ATV and rates facilities work together so that setting the rudder ATV to 80% in both directions and also setting a rudder rate of 60% will give a total rudder throw of 0.8 x 0.6 = 0.48 = 48%

µ §

Figure 3. The built-in Exponential of the ICG360 gives this response to rudder control

Automatic Tail Stabilisation (ATS)

In Heading Lock mode the ICG360 requires NO ATS. Use of ATS in this mode will give unwanted heading changes. Similarly, no Throttle-to-Tail Rotor mixing should be used in this mode. In Standard mode you may find very small amounts of ATS or Throttle-to-Tail Rotor mixing useful.

Pilot Authority Mixing

This should not be used with the ICG360 gyro.

Tail rotor dynamics

To realise the full benefit of the ICG360 it is important to have a basic understanding of the dynamics of the tail rotor system. It is a common misbelief that the higher the electronic gyro gain is, the better the system will work. Whilst this is generally true, the electronic gain is only one part of the tail rotor system and of equal importance is the amount of tail pitch range available, and the tail rotor disk size. The disk size is also related to the tail rotor speed. Of course there are other influences on the tail system but these are the most important and can be grouped together as mechanical gain.

If the mechanical gain is not correct, then the gyro will not be able to function to its best ability. For this reason it is important to select the correct size servo arm that will give maximum tail pitch movement without stalling the servo. Note: once this has been setup, increasing the rudder channel ATV/TA will not increase the servo throw as this is limited electronically by the gyro. The rudder ATV/TA is used instead to adjust the rotation rate.

Tail blade size

The correct length of tail blade can only be selected by flight testing. There are 2 types of test to ascertain the correct size:

1) With the model in the hover at its normal flight rpm, making sure the gyro is in heading hold mode, make a rapid vertical climb for 50 to 60 feet: there should be no discernible rotation of the model. If the tail does not maintain its position this is a good indication that the diameter is too small. Note: if the model is over pitched and there is a noticeable drop in rotor rpm during the climbout, then this will invalidate the test as the tail would probably not be able to cope with this situation regardless of the setup.

2) This test is only really necessary for those who wish to do advanced aerobatics/3D flying and should only be carried out those who are confident to do so. With the model at a safe height fly the model sideways, starting slowly and building up speed until it reaches its maximum possible speed:

a)If the tail manages to hold its position then the blade length is OK.

b)If the tail starts to lag behind, then either more pitch is needed (if the mechanical limits haven't already been reached) or longer tail blades.

c) If the tail swings round suddenly and cannot return to the correct position then the tail rotor is stalling and longer tail blades are needed.

Maximising Performance for Advanced 3D/Aerobatic Flight - by Bob Johnston

To get the maximum performance from this or any piezo gyro it is essential to minimise the vibration reaching the gyro sensor. Apart from the obvious balance checks, make sure that the canopy, fins, tuned pipe or muffler are not too loosely mounted, especially if the gyro is being used with a fast servo, as it is possible for a resonance to build up which shows as a very fast shake, visible in the canopy and tail.

If you feel confident to do so, try slowly tumbling the model while checking for any visible vibration, a small amount is almost inevitable, but try to dampen any large vibrations present. Alternatively tumble or roll the model at a safe height and if the gyro can be made to wag through the manoeuvre this is a good indication that a component is resonating.

To set the maximum gain permissable for aerobatic  flight, either a) fly the model as fast as possible in a straight line and twitch the rudder stick a small amount, if this causes the gyro to wag, lower the gain and repeat the test until the tail cannot be made to wag anymore. Obviously if the model cannot be made to wag initially, keep increasing the gain until it does, and then reduce the gain a few percent. Setting the gyro this way should prove sufficient for all flying except in very windy turbulent conditions where the gain may have to be lowered by a few percent. b) Alternatively, if you feel confident enough in your ability and your models reliability, put the model in a vertical dive for 3 or 4 hundred feet (90 to 120 metres) with the main rotor pitch at zero. Once terminal velocity has been reached, pull the model out in a smooth arc, and if the model does not wag under these conditions it is unlikely to do so under any conditions.

WARNING: Only attempt to carry out these flight tests if you are 100% confident in your own flying ability, and the structural integrity of your equipment.

Boom supports

Tests have shown that the use of boom supports significantly increase the rigidity of the tail boom and so allow a higher gyro gain to be used. Generally the longer the boom supports the better they will work. Note: A loose/cracked boom support or a loose/cracked boom support clamp can have a significant effect on the gyro gain.

Temperature stability

Although the gyro is temperature calibrated small temperature differences between the piezo sensor and the temperature sensor occur from internal heating of the piezo sensor. These may cause a small trim change during the first two minutes of operation. It is best to avoid flying the model immediately after exposure to sudden temperature changes such as removing it from a cool car to a very hot flying site.

Humidity

Under conditions of extremely high humidity it is possible for condensation to form on the piezo sensor. Formation and evaporation of this condensation will cause a shift in the gyro trim. Where it is not possible to avoid subjecting to the gyro to such humidity levels it is advisable to leave the model standing for a few minutes with the radio switched on so that the internal heating of the sensor disperses the condensation. Having done so, cycle the receiver power to reset the gyro. Always protect the gyro from ingress of water (e.g. rain).

PC interfacing

Your ICG360 is equipped with a 'COMPUTER' port which is provided for the experienced pilot wishing to optimise the internal settings of the ICG360's two flight modes to their helicopter and flying style. An optional Interface cable and software disk is available for use with this port. The interface/software is compatible with any IBM PC compatible with a parallel (printer) port and VGA graphics. Comprehensive additional instructions on the use of the PC interface are supplied with the software. Connection to the 'COMPUTER' port of the ICG360 must only be made via the correct optional interface cable.

No other form of connection to this port should be made as mis-use of this facility may damage the gyro unit.

Ratings

Weight: 					40g

Dimensions  : 					42.5mm High x  26.5mm Wide x 46.5mm Long

(excluding cables and mounting ears)

Power Supply: 					3.5 to 7.2v

Repairs & Servicing

For repairs and servicing only, please contact:-

Westbury Products

Armour House

Brunswick Square

Southampton

Hants

SO14 1AR     (Fax 01 703 335 463)

Manufactured in the UK by:

CSM Design Consultancy Ltd

8 Littlebrook Close

Hadfield

Hyde

Cheshire

SK14 8AW	(Fax: 0161 929 5984)

Distributed world-wide by:-

J. Perkins (Distribution) Ltd

90-96 Greenwich High Road

London SE10 8JE

(Tel: 0181 692 2451

Fax: 0181 692 2469)

DOs and DON'Ts

DOs:

Do mount the gyro with its axis of rotation parallel to the helicopter main shaft.

Do mount the gyro on two of the supplied self-adhesive foam strips.

Do mount the gyro to a hard, smooth clean surface.

Do use the rudder ATV and rates to tailor required stick response.

Do check the sense of the gyro operation and fit/remove Reversing link as needed before flight.

Do use a battery state monitor and check it before each flight.

Do remove slop and stiffness from tail control linkage.

Do inspect tail gears etc. for wear.

Do explore the performance limits of this gyro with care.

DON'Ts:

Don't mount the gyro where it will be subjected to high vibration levels.

Don't use Pilot Authority Mixing.

Don't use ATS or Throttle-Tail mixing with the Heading Lock mode.

Don't use unnecessarily long servo extension leads with the gyro.

Don't use Aux3 as gain channel with JR ZPCM receivers

Don't connect a servo extension to the "COMPUTER" port of the gyro.

Don't move the model during gyro self-test time.

ICG360 PC INTERFACE DOCUMENTATION

INTRODUCTION

The ICG360 gyro is equipped with a 'computer' port that allows the internal settings of the gyro to be adjusted to the users' requirements. Although for many users the default settings of the ICG360 (with which it is supplied pre-loaded) will be satisfactory, with careful experimentation, it may be possible to tailor the operation of the gyro to match more closely the particular model, radio equipment, and flying style of the pilot. Should you be unsuccessful in finding a better set-up, the PC software includes a facility for returning the gyro back to the factory default settings.

INSTALLING THE INTERFACE SOFTWARE ON YOUR PC

The installation program may be run under DOS or windows. The following description is for DOS. To install the gyro interface software onto a hard drive, insert the distribution disk in drive A  (or B) and at the DOS prompt type:-

A:		(Enter)  	{or B: if appropriate}

INSTALL	(Enter)

You will be asked which hard drive you want the software installed onto (e.g. "C"), and the floppy drive from which the software will be installed ("A" or "B"). Once the selection of drives has been confirmed, a directory called 'ICG360' will then be created on the appropriate drive, and the required files transferred. If the install program finds a previously installed version it will ask if you wish to overwrite it.

RUNNING THE INTERFACE SOFTWARE

Once the software has been installed on your PC follow the following steps to connect to your gyro.

1. CONNECT THE GYRO INTERFACE CABLE TO YOUR PC

µ §

Locate the Parallel (Printer) port of your PC. This is a 25 pin 'D' connector with socket contacts. If you have a printer attached to your PC it may already be connected to the desired port. If so, disconnect it and connect the gyro interface cable in its place. If your PC has multiple parallel ports you can install the interface on LPT1, LPT2 or LPT3. You will be able to select the port in use when running the interface program.

! DO NOT CONNECT YOUR PC TO THE GYRO YET !

2. START THE INTERFACE SOFTWARE

Do this by typing at the DOS prompt:-

GYRO   (Enter)

3. SELECT THE LPT PORT IN USE

At the first menu screen select the printer (LPT) port to which you have the gyro interface cable connected. If you are unsure of your LPT port's address you will need to try each option in turn until a successful connection to the gyro is achieved.

4. SELECT THE RC SYSTEM IN USE

At the next menu select the type of radio system you are using with the gyro. It is important to specify this correctly as some of the gyro internal variables are calculated based on the characteristics of the system in use.

5. TURN OFF TRANSMITTER (if present)

The gyro will only enter the PC interface mode if, during its turn on checks, it finds no servo pulses from the receiver. In order for this to happen your transmitter must be OFF.

6. TURN RECEIVER ON.

This will power up the gyro. After a short delay the gyro will start to flash its "SET" LED to indicate that it is in PC interfacing mode.

7. CONNECT INTERFACE LEAD TO "COMPUTER" PORT OF GYRO

This lead is easily identified as the only white/red/black ribbon cable emerging from the gyro.

8. PRESS ANY KEY TO INITIATE LINK-UP

TROUBLE SHOOTING

During the initial stages of the link up the integrity of the PC-gyro communications is checked and any problems will be reported. Failure to establish a link-up is usually due to the selection of the wrong LPT port address. As an aid the software automatically highlights your last port selection on being run.

NOTE FOR PPM (FM) RADIO USERS

You may find that, with the transmitter turned off, your PPM receiver picks up interference when close to your PC. This may cause problems in getting the gyro into PC interfacing mode and also cause unacceptable jitter of the servos (other than the tail servo) during PC link-up. If you have this problem proceed as follows:-

With your model well away from the PC turn on the receiver with the transmitter off. Observe the gyro "SET" LED and check that it is flashing. Now turn on your transmitter. Then bring the model close to your PC and complete the link-up as normal. The good signal from your transmitter will suppress the interference from the PC. Having finished with the PC link-up don't forget to turn off the transmitter. No such problems should arise with PCM receivers.

THE MAIN MENU

This offers the following options:-

Set up Flight Mode 0

Set up Flight Mode 1

Return Flight Modes to factory default settings

If you are familiar with using your ICG360 with its factory default settings then Flight Mode 0 is the so called 'Standard' mode while Flight Mode 1 is the 'Heading Lock' mode.

However, with the aid of this PC interface both Flight Modes are fully configurable and you could, if desired, set up both modes to have heading lock capability. This may be of particular interest for those using the ICG360 without the gyro gain input connected (and thus setting the gain of the gyro on the gain pot of the gyro). In this case only Flight Mode 0 is used. If desired, you can now set this mode to a Heading Lock mode. To do this simply copy the Factory Default values for Flight Mode 1 to Flight Mode 0.

The two Flight Mode setting sub-menus are identical and provide control over the following parameters:-

Conventional Gyro gain

This sets the degree of rate damping provided by the gyro, and is like the gain on an ordinary gyro.

Direct (Stick to servo) coupling

This provides the gain independent servo movement linked directly to the demanded yaw rate from the stick position. The setting of this parameter is discussed later.

Servo speed parameter

This relates to the speed of the servo travel and is in the units most commonly quoted by servo manufacturers (seconds for 60 degrees of servo travel). Remember, the smaller the number the faster the servo. The effect of this value may be affected by many things. Use the servo manufacturer's figure only as a starting point for your experimentation.

Heading-Lock gain

This sets the degree of resistance the gyro provides to unrequested heading changes. If you want a 'standard' (non Heading Lock) mode set the Heading Lock Range to zero.

Maximum yaw acceleration of helicopter

This parameter helps the gyro match its operation to the actual agility of the helicopter in yaw.  The value relates to the maximum achievable (or desired) rate of acceleration and deceleration of the helicopter in yaw. Helicopters with large yaw inertia (i.e. those with scale bodies) will generally have lower yaw acceleration than, say pod-and-boom designs. Setting a value that is higher than the helicopter can achieve will mis-match the gyro to the helicopter and degrade the quality of the start/stop transients. For scale flying you can set this parameter to a value lower than the helicopter can achieve to deliberately slow the response of the model in yaw, making for a more scale-like performance.

Heading Lock Range

This limit sets the range of headings over which the heading lock gain applies. It can, if desired be used in conjunction with the Heading Lock Gain to limit the maximum power of the Heading lock. Important: Set this parameter to zero for a 'standard' (non Heading Lock) mode.

Servo travel limit

This provides a way of preventing over-travel of the tail control linkage.

General remarks

Given the number of variables within the gyro as well as the many aspects of the helicopter (tail blade size, engine performance, servo speed, etc.) that influence the characteristics of the helicopter in yaw it is not possible to give a simple recipe for optimising the gyro for you and your model. However, we strongly recommend you start by getting your model performing well with the factory default settings. You should then try making small adjustments to the default settings and observe their effects. You may, for example wish to experiment with reducing the servo travel limit while increasing the servo arm length and observing if you get an improvement in the crispness of the start and stop transients.

In arriving at the factory default settings we found it useful to set the two modes to nearly identical settings differing only in, say, the value of  the servo speed parameter. By switching between the two modes we were able to see quite small changes and 'home in' on the optimum. Careful note-taking is vital to prevent confusion in this process! An observer taking notes on a clip board is very useful. Once you have used this technique to get one good mode (say a heading lock mode) you can then apply the same method to optimise your second flight mode.

As an example of the flexibility of the ICG360 system, a useful flight mode, especially for the beginner, is one  with only a small amount of Heading Lock (say with a Heading Lock gain of between 5 to 10% and a Heading Lock range of between 10 and 20 degrees). This mode provides adequate heading lock to maintain trim in light cross-wind hovering while, in forward flight  allowing the helicopter to weathercock should a turn not be well co-ordinated.

Adjustment of the Direct (stick to servo) Coupling can be left until other aspects of your Flight Modes have been set. The other parameters can be adjusted with the Direct Coupling set to the factory default value. Adjustment of the Direct Coupling is done by observing the effect of gyro gain changes on the maximum yaw rate. If you find that reducing the gyro gain causes the maximum yaw rate to decrease then more Direct Coupling is needed. Conversely, an increased maximum yaw rate at reduced gyro gain means that too much Direct Coupling is being used.

µ §